IH型单吸单吸离心化工泵

安装使用说明书

长沙福斯水泵泵业有限公司

Fuchs Changsha Pump pump industry limited company

概述 Instruction

IH型单级单吸悬臂式化工离心泵是取代F型耐腐蚀离心泵的节能、换代产品，该产品广泛用于化工、石油、冶金、造纸、食品、制药、合成纤维等工业部门，用来输送腐蚀性的或不允许污染的年度类似水的液体。泵的进口压力不允许超过0.3MPa。

IH型泵输送介质的温度一般为-20℃~105℃，必要时采取适当的冷却措施，可输送更高温度的液体。

Type IH pump is single-stage, single-suction chemical cantilever centrifugal pump, it is instead of type F anti-corrosive centrifugal pump, it is a saving energy products, it widely used in the field of chemical engineering, petroleum, metallurgy paper-making, food, medicine, fiber, etc., it is suitable for delivery corrosive medium or the medium which are not permitted to contaminate, the inlet pressure is ≤0.3MPa, the medium temperature is 20℃ to 105℃, it can deliver more higher temperature medium by cooling unit.

型号意义 Type Description

IH50-32-160A-303

IH-国际标准化工泵                  International Standard Chemical centrifugal pump

50-泵吸入口直径为50mm            Inlet diameter is 50mm

32-泵排出口直径为32mm            Outlet diameter is 32mm

160-叶轮名义直径为160mm                Nominal impeller diameter is 160mm

A-叶轮外径切割代号（A、B分别表示第一、第二次切割）     Impeller first cut

303-泵触液零件的材料代号：ZG1Cr18Ni9          Material codes: ZG1Cr18Ni9

性能范围（设计点）         Performance Range(design point)

流量：6.3~400m3/h                   Flow:6.3~400m3/h

扬程：5~125m                       Head:5m~125m.

结构特点 Construction Feature

IH化工泵主要零件有泵体、叶轮、密封环、叶轮螺母、泵盖、密封部件、中间支架、轴和悬架等。

泵的结构特点是：泵盖通过止口固定在中间支架上，然后通过泵体与中间支架止口的联接，把泵盖夹紧在中间。泵体是轴向吸入、径向排出、底脚支承式，可直接固定在底座上。悬架部件通过止口固定在中间支架上，并用悬架支架支撑在底座上。检修时可不拆卸进出口联接管路、泵体和电机，只需拆下加长联轴器的中间联接件，即可退出转子部件进行检修。

轴封有填料和机械密封等9种形式，可根据输送介质的性质和生产工艺的要求进行选定。

泵通过加长联轴器与电机联结，从驱动端看，泵为顺时针方向旋转。

Main parts of type IH chemical pump are pump casing, impeller, wearing impeller nut, pump cover, sealing parts, intermediate support bearing, and cantilever etc.

The construction feature: pump cover is fixed on intermediate support, when pump casing is axial suction and radial discharge, it adopt foundation support, the casing can fixed on foundation base directly, cantilever part fixed on intermediate support, born by foundation base. In repair, only disassemble the mid-coupling, the rotor can be disassembled to repair, it needn’t disassemble the motor, pump and pipe.

The packing seal and mechanical seal have 9 kinds, it can be selected according medium or manufacturing technology, the pump couple the motor by lengthening coupling. View from motor end, the rotating direction is clockwise.

材料 Material

根据液体的腐蚀情况，可选用相适应的泵过流部件的材质。触液零件材质如下表：

According to medium causticity, the material can be selected as follows:

| 材质material | ZG1Cr18Ni9 | ZG1Cr18Ni9Ti | ZG0Cr18Ni12Mo2Ti | ZG1Cr18Ni12Mo2Ti |
| --- | --- | --- | --- | --- |
| 代号code | 303 | 305 | 306 | 307 |
| 材质material | ZG00Cr14Ni14Si4 | ZG00Cr18Ni10 | Cr28 | ZG00Cr17Ni14Mo2 |
| 代号code | 300 | 301 | 203 | 316L |

结构图 Construction Drawing

1、泵体      Pump casing

2、叶轮      Impeller

3、密封环    Wearing ring

4、叶轮螺母  Impeller

5、泵盖      Pump cover

6、密封部件  Seal parts

7、轴套      Shaft sleeve

8、中间支架  Intermediate support

9、轴        Shaft

10、悬架部件 Cantilever part

轴封结构图

Shaft Seal Construction Drawi

P3带填料环的软填料密封

P1软填料密封                        P2带填料环的软填料                密封通常还有喉部衬套

P1 soft packing                        P2 soft packing with packing ring         P3 soft packing with packing ring there

is a throat lining

S3有旋转密封环的(啮合环)的

S1内装式装置                        S2外装式装置                      内装式装置

S1 the internal unit                     S2 the external unit                   S3 the internal unit with rotating

seal packing ring (engaged ring)

D3面对面装置可装置旋转密

D1背靠背装置                       D2串联装置                        封环(啮合环)

D1 back to back unit                   D2 series around unit                  D3 face to face unit with rotating

seal packing (engaged ring)

型 谱 图

Available head-capacity curves

性能参数表

Sheet of Performance Date

性能参数表

Sheet of Performance Date

性能参数表

Sheet of Performance Date

性能参数表

Sheet of Performance Date

性能参数表

Sheet of Performance Date

性能参数表

Sheet of Performance Date

性能参数表

Sheet of Performance Date

性能参数表

Sheet of Performance Date

性能参数表

Sheet of Performance Date

订货须知：

- 应按使用条件决定泵的型号、规格、材料、密封形式及原动机。在订货时应提供介质名称、流量、扬程、吸入压力、密度、浓度、温度等。并指出介质的易燃性、易爆性、毒性、腐蚀性、结晶性以及是否封液渗入介质内等。

- 若对原动机有特殊要求时用户应说明现场条件，环境温度和湿度，安装地点及类别（防爆、防火要求）。

Note:

1. According to the user condition, select pump type, size, material, seal type, and driver. In order, the user should state the medium, flow, head, suction pressure, gravity temperature etc. , and the medium fire, corrosive, etc. feature.

2. If the driver have special requirement, the user should state spot condition, ambient temperature, etc. (ex-proof, fire-proof requirement)

外形尺寸图

Overall Dimension Drawing

外形安装尺寸图

Overall installing Dimension Drawing

外形尺寸表

Overall Dimension Sheet

外形安装尺寸表

Overall installing Dimension Sheet

外形安装尺寸表

Overall installing Dimension Sheet

外形安装尺寸表

Overall installing Dimension Sheet

外形安装尺寸表

Overall installing Dimension Sheet

外形安装尺寸表

Overall installing Dimension Sheet

外形安装尺寸表

Overall installing Dimension Sheet

外形安装尺寸表

Overall installing Dimension Sheet

外形安装尺寸表

Overall installing Dimension Sheet

外形安装尺寸表

Overall installing Dimension Sheet

法兰尺寸图

Flange Dimension Drawing

法兰尺寸表

Flange Dimension Sheet

| 泵型号 Pump type | 吸入口法兰尺寸(mm) suction flange dimension |  |  |  |  |  |  | 排出口法兰尺寸(mm) discharge flange dimension |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DN1 | D1 | D11 | d1 | b1 | f1 | n1-d01 | DN2 | D2 | D12 | d2 | b2 | f2 | n2-d02 |
| IH 50-32-125 | 50 | 165 | 125 | 102 | 20 | 3 | 4-φ17.5 | 32 | 140 | 100 | 78 | 18 | 2 | 4-φ17.5 |
| IH 50-32-160 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IH 50-32-200 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IH 50-32-250 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IH 65-50-125 | 65 | 185 | 145 | 122 | 20 | 3 | 4-φ17.5 | 50 | 165 | 125 | 102 | 20 | 3 | 4-φ17.5 |
| IH 65-50-160 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IH 65-40-200 |  |  |  |  |  |  |  | 40 | 150 | 110 | 88 | 18 | 3 | 4-φ17.5 |
| IH 65-40-250 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IH 65-40-315 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IH 80-65-125 | 80 | 200 | 160 | 133 | 22 | 3 | 8-φ17.5 | 65 | 185 | 145 | 122 | 20 | 3 | 4-φ17.5 |
| IH 80-65-160 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IH 80-50-200 |  |  |  |  |  |  |  | 50 | 165 | 125 | 102 | 20 | 3 | 4-φ17.5 |
| IH 80-50-250 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IH 80-50-315 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IH 100-80-125 | 100 | 220 | 180 | 158 | 24 | 3 | 8-φ17.5 | 80 | 200 | 160 | 133 | 22 | 3 | 8-φ17.5 |
| IH 100-80-160 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IH 100-65-200 |  |  |  |  |  |  |  | 65 | 185 | 145 | 122 | 20 | 3 | 8-φ17.5 |
| IH 100-65-250 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IH 100-65-315 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IH 125-100-200 | 125 | 250 | 210 | 184 | 26 | 3 | 8-φ17.5 | 100 | 220 | 180 | 158 | 24 | 3 | 8-φ17.5 |
| IH 125-100-250 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IH 125-100-315 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IH 125-100-400 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IH 150-125-250 | 150 | 285 | 240 | 212 | 26 | 3 | 8-φ22 | 125 | 250 | 210 | 184 | 26 | 3 | 8-φ17.5 |
| IH 150-125-315 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IH 150-125-400 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IH 200-150-250 | 200 | 340 | 295 | 268 | 30 | 3 | 12-φ22 | 150 | 285 | 240 | 212 | 26 | 3 | 8-φ22 |
| IH 200-150-315 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IH 200-150-400 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

装配与拆卸

- 泵的拆卸顺序

  - 拧下泵体上的放液管堵和悬架体上的放油管堵，放净泵内液体及悬架体内储油室的存油。

  - 拆开泵体与中间架的联接螺栓，将中间支架、悬架部件、密封部件等全部转子组件从泵体中取出（在此之前先拆出加长联轴器的中间联接件）。

  - 松开叶轮螺母，取出叶轮和键。

  - 将泵盖连同轴套，机械密封端盖及机械密封等组合件一起从轴上取下。此时应注意勿使周涛与泵盖等相对滑动。然后再把机械密封端盖拆开。将机械密封连同轴套一起取出，再把周涛和机械密封拆开。如果是软填料密封，可先从泵盖上取下轴套，在顺次拆开填料压盖、填料和填料环等。对特殊结构的机械密封，要注意其不同的拆装方法。

  - 拆下中间支架和悬架支架。

  - 拆下泵联轴器和键。

  - 拆下悬架两端的防尘盘和轴承的前、后盖，再把轴连同轴承一起从悬架中取出。

  - 拆开轴承和轴。

- 泵的装配顺序

泵的装配顺序基本上可按照拆卸顺序的反向进行，但装配时要注意检查个密封面垫片应完好，并注意切勿漏装垫片和更换部完好的垫片。

Assembly and Disassembly

- Pump Disassembly Sequence

  - Screw off releasing water stopper on pump casing and releasing oil stopper on cantilever support, release out the water and oil. (At first disassemble sealing water pipe. )

  - Disassemble the joints between casing and intermediate support, cantilever part, sealing part, rotor, etc.. (Disassemble intermediate coupling at first. )

  - Screw off the impeller nut, pull out impeller and key.

  - Pull out pump cover, shaft sleeve, seal end cover, mechanical seal etc. .from shaft together, preventing sleeve and cover sliding. Then disassemble seal end cover, pull out mechanical seal together with sleeve, then disassemble mechanical seal and shaft sleeve. If it is packing seal, shaft sleeve can be pull out from pump cover at first, then disassemble packing cover, packing, packing ring, etc. in orderly. For special construction mechanical seal, adopt relevant support disassembling method.

  - Disassemble intermediate support and cantilever support.

  - Disassemble pump coupling and key.

  - Disassemble dust proof disc, in the two sides of cantilever support and fort & rear cover of bearing, then pull out shaft and bearing from cantilever support together.

  - Disassemble bearing and shaft.

- Pump assembly sequence

Assembly is according to the opposite order of disassembly, but in assembly, the seal gasket must ensure well. Don’t lose gasket or adopt bad gaskets.

安装

- 开箱后检查泵和电机，如果证实没有任何因装、卸和运输过程中造成的损坏和紧固联节件松动，泵的进、出口封盖完好，没有尘土、污物进入泵内，则可不必重新拆卸和装配，直接送到使用现场进行安装。

- 安装泵的基础面应用水平仪找平。待基础水泥凝固后，将泵安装在基础上，并用水平仪检查泵和电机轴的水平状况。如果不水平，应用垫铁调整，直到水平为止。然后用水泥浇灌底座和地脚螺栓孔眼（底座的地脚螺栓一般留预留孔，采用二次灌浆埋设）。

- 水泥干固后，应检查底座和地脚螺栓是否松动，合适后拧紧地脚螺栓配套螺母，重新检查水平度。

- 在电机、泵和底座重新安装情况下，应严格泵轴和电极轴的同心度。测量联轴器的外圆上、下、左、右的差别不超过0.1mm，两联轴器面间隙在一周范围内最大的最小的间隙差不得超过0.3mm，名义间隙2~3mm。

- 泵的吸入管路和压出管路应有各自的支架，不允许管路的重量直接由泵来承受，以免把泵压坏。

- 泵的安装位置高于液面（在泵的吸程允许范围内）时，应在戏入管路端部装上底阀，并在排除管路上设置灌液螺孔或阀门，供起动前灌泵之用。泵的安装位至低于液面（灌注情况）时，应在吸入管路上装上控制阀门和过滤装置，以防杂物吸入泵内。

Installing

- After opening packing case, check motor and pump, ensure the pump and motor well and clean them, the pump should not be disassembled, or reassembled it will send to spot to install directly.

- Foundation surface of installing should be leveled by gradienter, after the grout is hard, place pump on foundation, check the pump & motor shaft level by gradienter, if the foundation is not level, adopt wedges to adjust, till the foundation to level, then pour foundation and foundation bolt holes with concrete (foundation bolts usually have ready holes, second pouring to fill in.)

- After concrete dry, check foundation bolts and ensure it tighten. Tight the nuts on the foundation bolts, check level degree again.

- If the pump set is reinstalled, the concentricity should be check strictly, measured the tolerance value is less than 0.1mm, the end face clearance is 2~3mm.

- The inlet pipe outlet pipe should have itself support, the pump don’t allow to bear pipe weight, to avoid to be crushed.

- If installing height is over the liquid level, the foot valve should be install in the suction pipe, and install pouring water bolt hole or valve, in discharge pipe to use for pouring water into pump before start, if the installing height is low than the liquid level, controlling valve and strainer should be installed on suction pipe, to prevent mess going to pump.

使用

- 起动

1.1 准备必要的工具

1.2 检查悬架体储油室油位，应控制在油位计中心线2mm处左右的位置上。

1.3 检查电动机的转动方向是否正确，严禁反转。

1.4 用手转动联轴器，应感觉轻松且轻重均匀。注意辨别泵内有无磨擦声和异物滚动等杂音。如有，则应设法排除，并将联轴器的防护罩安好。

1.5 泵的安装位置低于液面（灌注情况）时，起动前应打开吸入管路的闸阀，使液体充满泵内。如泵的安装位置高于液面（真空情况），起动前要灌泵或抽真空，使泵内和吸入管内充满液体，排净泵内空气。

1.6 关闭进出口压力（或真空）计及压出管路闸阀，起动电机（最好先点动，确认泵转动方向正确后再正式运行），打开进出口压力表，再慢慢打开出口管路闸阀到所需要的位置。

2. 运转

2.1 要经常检查泵和电机的温升情况，轴承的温升不应大于35℃，极限温度不应大于75℃。

2.2 应注意悬架体储油室油位的变化，经常控制在规定范围内，为了保持油的清洁和良好的润滑，应根据现场使用的实际情况，定期更换新油，一般情况，每运转1500小时后，要全部更换新油一次。

2.3 在运转过程中，发现有不正常的声音和其他故障时，应立即停车检查，待故障排除后才能继续运转。

2.4 绝不允许用吸入管路上的闸阀来调节流量，以免产生汽蚀。

2.5 泵一般不低于30%设计流量下长期运转，如果必须在该条件下使用，应在出口管路上安装旁通管，使泵的流量达到规定的使用范围。

3 停车

3.1 慢慢关闭压出管路闸阀，停止电机。

3.2 关闭进出口压力（真空）计，灌注情况下还要关闭吸入管路闸阀。如果密封采用外部引液时，还要关闭外引液阀门。

3.3 如环境温度低于液体凝固点时，要放净泵内液体，以防泵冻裂。长时间停止使用的泵，除将泵内腐蚀性液体放净外，还要用清水洗净，尤其是密封室要认真冲洗干净。最好将泵拆下清洗后重新装好，并将泵的进出口封闭后妥善保管。

Operating

1 Starting

1.1 Prepare necessary tools.

1.2 Check oil level on cantilever support, the oil level should be on the position of centerline about 2mm.

1.3 Check the rotary direction, the motor don’t allow to rotate in opposite direction.

1.4 Turning the coupling, it should be flexible, and not noise. Then install coupling proecting cover.

1.5 If the installing level is low than liquid level, before start, the suction pipe valve must open, to make pump fill in water. If the pump installing level is over than liquid level, before start, the pump must be poured water or pump vacuum, to ensure the pump filling in liquid without air.

2 Running

2.1 The pump & motor temperature rise should be usually checked, bearing temperature rise is not more than 35℃, max temperature is not more than 75℃.

2.2 Check the oil level in oil chamber, control the oil level in required range, to ensure oil clean and good lubricating. According to the spot condition, renew oil. The oil should be renewed after running 1500 hours.

2.3 During the running, the noise is not normal or other trouble, it should be stop to check, after shooting these trouble the pump can go to run again.

2.4 Don’t allow to adjust flow by fate valve on suction pipe, to avoid cavitation.

2.5 The pump don’t allow to run long time at flow ≤30% design point, if the pump must be run in these working points, install branch pipe on the discharge pipe, to ensure flow in required range.

3 Stopping

3.1 Slowly switch off the fate valve on outlet pipe, then stop motor.

3.2 Switch off inlet & outlet pressure meter, switch off fate valve on suction pipe if the pump suction is poured. If sealing water is outer water, the valve on the system should be switch off.

使用机械密封注意事项

本系列泵可根据使用条件不同，安装不同形式的机械密封，加内装单端面平衡型和非平衡型，双端面平衡型及非平衡型，外装式机械密封等等。因此，根据所选用机械密封形式的不同，其使用方法和注意事项也有所不同，具体情况应按照《机械密封安装使用说明书》中的规定处理，下面仅提几点一般应注意的事项：

  - .一般机械密封适用于清洁的、无悬浮颗粒的介质中。因此，对新安装的管路系统及储液罐，应认真冲洗干净，严防固体杂质进入机械密封面而使密封失效。

  - 在易结晶的介质中使用机械密封时，要注意经常冲洗。重新起动前，要将机械密封上的结晶冲洗干净。

  - 拆卸机械密封要仔细，不许用手锤、铁器等敲击，以免破坏动、静环密封面。

  - 如果有污垢拆不下来时，不要勉强去拆，应设法清除污垢，冲洗干净后，再进行拆卸，以免损坏密封元件。

  - 安装机械密封前，应检查所有密封元件是否有失效和损坏的，如有则应重新修复或更换新元件。

  - 应严格检查动环与静环的对磨密封端面，不允许有任何细微的划伤、碰破等缺陷。所有零部件，包括泵体、叶轮、密封腔等在装配前均应冲洗干净，尤其是动、静环端面，要用清洁、柔软的布或棉纱认真擦拭干净，然后涂上一层清洁的油脂或机油。

  - 装配中要注意消除偏差，紧固螺钉时要均匀把紧，避免发生偏斜，使密封失效。

  - 正确调整弹簧的压缩量，使其不致太紧或太松。太紧，密封端面很快磨损而失效，而且消耗功率大；太松，则密封不起作用，泄漏量过大。所以，泵安装好后，以手扳动转子时，应感觉到密封弹簧有一定的压缩量，又能轻快、灵活地转动没有咬紧的感觉。如感觉太紧或盘不动，则应适当调松一些。

  - 对由外部冲洗的机械密封，起动前应先开启冲洗液使密封腔内充满密封液。停车时，先停泵，后关密封冲洗液。

  - 根据JB/T4127.1-1999《机械密封技术条件》，机械密封内径≤50mm，泄漏量≤3ml/h，内径＞50mm，泄漏量≤5ml/h，被密封介质有腐蚀性时寿命为0.5~1年。使用条件苛刻时不受此限。

How to use mechanical seal

The serial pump adopts different mechanical seal according to different condition, for example the internal single end face balance type and no-balance type, double end face balance type and no-balance type, the external mechanical seal etc.. so different mechanical seal have different using & installing method and attentive item, specific condition are disposed according to the requirement in “Mechanical Seal Installing & using manual”. Mere common attentive item is supplied as follow:

- Common mechanical seal is suitable for clear and no suspending solid medium, so the new installing pipe system and storing tin should be clean. Prevent to solid grain to go to sealing face of mechanical seal to make mechanical seal invalidation.

- To medium which is crystallized easily, the mechanical seal should be usually washed. Before starting again, the crystal should be cleaned.

- In dismounting the mechanical seal, don’t allow to hammer using hammer and ironware etc. to avoid damaging static ring or dynamic ring sealing face.

- If it have dunghill in mechanical seal, it is hard to dismount, don’t force to dismount, after riding the dunghill, clean it, then dismount, avoiding the seal elements are damaged.

- Before mounting the mechanical seal, check the seal elements, if it have some trouble, repair or renew the mechanical seal.

- Strictly check relative sealing end face of static ring and dynamic ring, don’t allow any small laceration or disfigurement, all parts, include pump casing, impeller, sealing chamber, should cleanup. To static ring or dynamic ring end face specially, adopt clean and soft cotton yarn to cleanup, then paint a lay clean grease or oil.

- In mounting mechanical seal, pay attention to eliminate the tolerance, tighten the bolts equally, avoiding deflection to make it invalidation.

- Correctly adjust spring compress valve, ensure it flexibly, if it is too tight or too loose, to adjust it.

- To mechanical seal with outer water cooling and washing, before starting, switch washing water to fill in the sealing chamber. In stopping, stop pump at first, then switch off the washing water.

- According to JB/T4127.1-1999 “Mechanical Seal Technology Condition”, internal dia. of mechanical seal ≤50mm, the leakage flow ≤3ml/h, the internal dia. of mechanical seal ＞50mm, the leakage flow ≤5ml/h, the mechanical seal life is 0.5~1 year in corrosive medium.

可能发生的故障、原因和消除方法

发生振动及杂音                                       轴封腔泄漏过多

消耗功率过大                                         轴封(包括填料函)发热

流量、扬程不足                                       泵过分发热或转不动

泵输出液体不足                                       轴承发热或容易损坏

| 故障的可能因素 |  |  |  |  |  |  |  |  | 消除办法 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 泵内或吸入管内留有空气 | ○ | ○ |  | ○ |  |  |  |  | 重新灌液，排除空气 |
| 吸上扬程过高或灌注不足 | ○ | ○ |  | ○ |  |  |  |  | 降低泵位，增加进口处压力 |
| 吸入管路过小或有杂物堵塞 | ○ | ○ |  | ○ |  |  |  |  | 加大吸入管径，清除堵塞物 |
| 吸入管浸入深度不够或漏气 | ○ | ○ |  |  |  |  |  |  | 增大浸入深度或检修管路 |
| 转速过高或过低 | ○ | ○ | ○ |  |  |  |  |  | 检查原动机转速，按要求规定 |
| 泵转向不对 | ○ | ○ |  |  |  |  |  |  | 从原动机方向看应为顺时针转 |
| 装置扬程与泵扬程不符 | ○ | ○ | ○ |  |  |  |  |  | 设法降低吐出系统阻力或高度 |
| 介质重度与粘度与泵的要求不符 | ○ | ○ | ○ |  |  |  |  |  | 重新核算或更换合适功率电机 |
| 在流量过少处运转产生振动 |  |  |  | ○ | ○ |  | ○ | ○ | 加大流量或设旁通循环管 |
| 泵与电机轴线不一致或轴弯曲 |  |  | ○ | ○ |  | ○ | ○ | ○ | 重新调整对中 |
| 转动部分与固定部分有严重摩擦 |  |  | ○ | ○ |  |  | ○ | ○ | 检修泵或改善使用工况 |
| 轴承磨损严重或损坏 |  |  |  | ○ |  | ○ | ○ | ○ | 更换新的 |
| 密封环磨损过多 |  | ○ | ○ |  |  |  |  |  | 更换新的 |
| 轴套填料或动静环磨损过多 |  |  | ○ |  | ○ | ○ | ○ |  | 更换新的 |
| 填料或机械密封选用或安装不当 |  |  | ○ |  | ○ | ○ | ○ |  | 按使用要求重配或安装 |
| 转动部分失去平衡引起振动 |  |  |  | ○ | ○ | ○ | ○ | ○ | 检查原因设法消除 |
| 悬架体油室油量过多或油太脏 |  |  | ○ | ○ |  |  |  | ○ | 按油位计加油或换新油 |
| 管路或泵内有杂物堵塞 | ○ | ○ |  | ○ |  |  |  |  | 认真检查或排污 |
| 双端面密封液压力不当 |  |  |  |  | ○ | ○ |  |  | 压力应比轴封前大0.05~0.1MPa |
| 填料压力盖过紧或过松 |  |  | ○ |  | ○ | ○ |  |  | 适当调整之 |

Possible trouble & remedies

The pump vibrates heavily and unexpected noises                    too much leakage from shaft seal

Pump requires excessive power                                   shaft seal is heated

not enough capacity or too low head                               pump is overheated or vibrates heavily

No water is pumped out                                         Bearing is heated or damaged easily

| Possible causes |  |  |  |  |  |  |  |  | Remedies |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Have air in the pump or suction pipe | ○ | ○ |  | ○ |  |  |  |  | Pour water into the pump again, release air |
| Suction head too high or not enough poured | ○ | ○ |  | ○ |  |  |  |  | Reduce pump level, increase inlet pressure |
| Suction pipe is too small, or it is clogged | ○ | ○ |  | ○ |  |  |  |  | Enlarge suction pipe dia, clean the pipe |
| Suction pipe is not enough submerged height, or leakage of air | ○ | ○ |  |  |  |  |  |  | Increase submerged depth, or repair the pipe |
| Speed is too high or too low | ○ | ○ | ○ |  |  |  |  |  | Adjust the speed according the pump requirement |
| Rotating direction is not right | ○ | ○ |  |  |  |  |  |  | View from motor end, the rotating direction should be clockwise |
| Installation head have different with pump head | ○ | ○ | ○ |  |  |  |  |  | Reduce the discharge system resistance or height |
| Medium gravity and viscosity have different with pump itself required | ○ | ○ | ○ |  |  |  |  |  | Change right motor |
| Flow is too small to vibrate |  |  |  | ○ | ○ |  | ○ | ○ | Enlarge flow or install branch circulation pipe |
| Shaft of pump and motor is not coaxial line |  |  | ○ | ○ |  | ○ | ○ | ○ | Adjust centering degree again |
| Rotating part and static part are worn heavily |  |  | ○ | ○ |  |  | ○ | ○ | Check pump or mend working condition |
| Bearing is worn heavily |  |  |  | ○ |  | ○ | ○ | ○ | Renew the bearing |
| Wearing is worn heavily |  | ○ | ○ |  |  |  |  |  | Renew the wearing |
| Packing or static ring is worn out |  |  | ○ |  | ○ | ○ | ○ |  | Renew these parts |
| Packing mechanical seal is not right |  |  | ○ |  | ○ | ○ | ○ |  | Reassemble or reinstall according the requirement |
| Rotating part lose balance or vibrates |  |  |  | ○ | ○ | ○ | ○ | ○ | Found the cause and remedies |
| In cantilever oil chamber, the oil is too much, or the oil is dirty |  |  | ○ | ○ |  |  |  | ○ | Fill in oil according requirement or renew oil |
| Flow passage or pipe is clogged | ○ | ○ |  | ○ |  |  |  |  | Check and clean flow passage |
| Mechanical seal water pressure is too low |  |  |  |  | ○ | ○ |  |  | The pressure should be over 0.05~0.1MPa than sealing water |
| Packing cover is too tighten or too loose |  |  | ○ |  | ○ | ○ |  |  | Adjust it suitably |